Report For Lab 4

This lab was about prime path coverage. The unit under test was scanRegion(), which adds up the cells of a grid, turns the sum into a score, and uses a switch to pass the score to adjust(). The control flow graph has thirteen nodes and sixteen edges, with nested loops, a two-way branch and a switch. I worked in the order the lab sheet gives: first node coverage, then edge coverage, then the prime paths. In the end I wrote 11 GoogleTest cases that tour all 19 feasible prime paths, and all of them passed.

Infeasible prime paths. P11 (1-2-7-8-10-11-13) and P12 (1-2-7-8-10-12-13) go from node 2 straight to node 7, so the loops never ran and sum is still 0. Node 7 asks sum > 10, which is false for 0, so edge (7,8) can never be taken on these paths. P14 (1-2-7-9-10-12-13) also has sum = 0. Node 7 takes the false branch to node 9 correctly, but then the switch evaluates 0 % 3 = 0, which always goes to case 0 (node 11) and never to the default arm (node 12). So edge (10,12) cannot follow on this path. In all three cases the loop never runs, so the sum is fixed at 0 and the later decisions can't go the other way. I learned how to build a CFG from code and read prime paths off it. Edge coverage is stronger than node coverage in general, but on this small graph two node coverage tests already hit every edge, so the real difference shows up with prime paths. Prime path coverage also covers the loop cases, such as an empty row, a loop that runs once and a loop that runs several times, which node and edge coverage don't force. I also found that one test can tour several prime paths, which kept the number of tests down to 11. Finally, some prime paths are infeasible because of the data dependency on sum, so they can't be covered by any test.